# DEMO – Device configurations

Contents

Introduction	2

Scenario 1	2

MikroTik	3

3COM 4500G	3

Corporate WEB and DNS Server	4

Cisco 3560	4

Ruckus Wireless AP	5

Scenario 2	6

MikroTik	7

Corporate WEB, DNS and DHCP Server	8

Cisco 3560	8

Ruckus Wireless AP	9

# Introduction

This document provides a very high-level explanation of the device configurations used in the demonstration in Lecture 9. It does not provide step by step details and can be used just as a reference. The topologies and the detailed configurations are explained in the lecture. For the details, please have a look at the video for Lecture 9.

# Scenario 1

## MikroTik

- Bridges and interfaces

| Bridge name | Member interfaces | IP address | Details |
| --- | --- | --- | --- |
| Bridge | ether2, ether3, ether4 | 192.168.99.1 | DHCP server |
| Bridge-WiFi | wlan1 | Automatic (DHCP client) | SNAT (masquerading) has to be configured |
| N/A | ether1 | Automatic (DHCP client) | Default SNAT (masquerading) |
| Bridge-Port5 | ether5 | 10.5.5.1/24 10.10.10.1/24 | VLAN 5 VLAN 10 |

- Source NAT (masquerading)

  - From “any” to “Out interface: Bridge-WiFi”, action: “masquerade”

  - From “any” to “Out interface List: WAN” (default rule), action: “masquerade”

- Configuration of wlan1 as wireless client

  - Create a new security profile in Wireless Tables -> Security Profiles and configure it with a name, authentication type and password

  - Configure wlan1 in wireless mode: “station”, associate it with the security profile created before and scan for WiFi networks

- [Optional] Create ACL to block the routing between VLAN 5 and VLAN 10

Note: The steps in green are only used when the MikroTik requires wireless connection to the main home router (access point) and because of this acts as a wireless client. This is normally used only during the lab/demo preparation and not used in the actual demonstration. Instead, a wired/cable connection is normally used as it requires less configuration steps and is more reliable.

## 3COM 4500G

- Baud rate is 19200 (needed for the console connection)

- G1/0/1 and G1/0/2 should be trunks (do not forget to include all VLANs)

- G1/0/3 should be access port in VLAN 10

- IP addresses and default gateway are optional, but useful for testing

- DNS – entirely for testing purposes (“dns server 8.8.8.8” and “dns resolve”)

## Corporate WEB and DNS Server

- The VM inside the laptop uses “External” Hyper-V switch, bridged directly to the LAN card

- WEB and DNS roles are installed inside the VM

- The DNS has one zone - “softuni.local” (used only by a corporate/employee network) and has a forwarder to 8.8.8.8 for all other requests

- The two DNS A records (softuni.local and www.softuni.local) point to 10.10.10.101

## Cisco 3560

- VLANs, trunk ports and IP addresses should be created

- Default gateway is not required but useful for testing. Note the L2/L3 mode of the device

- DNS – entirely for testing purposes (“ip domain-lookup” and “ip name-server 8.8.8.8”)

- DHCP scopes and exclusions:

  - VLAN5 (ip dhcp pool VLAN5; network 10.5.5.0 /24; default-router 10.5.5.1; dns-server 8.8.8.8)

  - VLAN10 (ip dhcp pool VLAN10; network 10.10.10.0 /24; default-router 10.10.10.1; dns-server 10.10.10.101)

  - Ip dhcp excluded-address 10.5.5.1, 10.5.5.2, 10.10.10.1, 10.10.10.2, 10.10.10.101

## Ruckus Wireless AP

# Scenario 2

## MikroTik

- Bridges and interfaces

| Bridge name | Member interfaces | IP address | Details |
| --- | --- | --- | --- |
| Bridge | ether2, ether3, ether4 | 192.168.99.1 | DHCP server |
| Bridge-WiFi | wlan1 | Automatic (DHCP client) | SNAT (masquerading) has to be configured |
| N/A | ether1 | Automatic (DHCP client) | Default SNAT (masquerading) |
| Bridge-Port5 | ether5 | 10.23.23.1 | N/A |

- Source NAT (masquerading)

  - From “any” to “Out interface: Bridge-WiFi”, action: “masquerade”

  - From “any” to “Out interface List: WAN” (default rule), action: “masquerade”

- Configuration of wlan1 as wireless client

  - Create a new security profile in Wireless Tables -> Security Profiles and configure it with a name, authentication type and password

  - Configure wlan1 in wireless mode: “station”, associate it with the security profile created before and scan for WiFi networks

- Static routing. The device will do NAT to outside, but the Cisco 3560 in the inside network will route from different subnets, that is why a static route to 10.0.0.0/8 is needed (using ether5 or pointing to 10.23.23.3)

Note: Separate routes to 10.5.5.0/24, 10.10.10.0/24 and 10.13.13.0/24 will also work here.

Note: The steps in green are only used when the MikroTik requires wireless connection to the main home router (access point) and because of this acts as a wireless client. This is normally used only during the lab/demo preparation and not used in the actual demonstration. Instead, a wired/cable connection is normally used as it requires less configuration steps and is more reliable.

## Corporate WEB, DNS and DHCP Server

Changes from Scenario 1 include:

- DHCP role installed and configured with the following scopes:

  - VLAN5-Guests (Pool: 10.5.5.10-10.5.5.50, Default gateway: 10.5.5.3, DNS: 8.8.8.8)

  - VLAN10-Corp (Pool: 10.10.10.10-10.10.10.50, Default gateway: 10.10.10.3, DNS: 10.13.13.101)

- IP address of the VM (hosting the three services) changed to 10.13.13.101

- Default gateway of the VM changed to 10.13.13.3 (Cisco 3560)

- The two DNS A records (for softuni.local and www.softuni.local) changed to 10.13.13.101

## Cisco 3560

- VLANs, trunk ports and IP addresses should be created (use the “no switchport” command to configure IP addresses directly on Fa0/2 and Fa0/3)

- Default gateway is required. Note the L2/L3 mode of the device (L3 is required in this scenario)

- DNS – entirely for testing purposes (“ip domain-lookup” and “ip name-server 8.8.8.8”)

- DHCP relay for VLAN 5 and VLAN 6, pointing to 10.13.13.101

## Ruckus Wireless AP

This device has the same configuration as from Scenario 1.